Additional end-of-chapter problems for Chapter 4 – Digital Filters

CMOS: Mixed-Signal Circuit Design

A4.1 	Using LTspice regenerate the 2- and 4-bit equivalents of Fig. 4.1. The y-axis will be voltages in decimal format.

A4.2	Using LTspice regenerate the 2- and 4-bit equivalents of Fig. 4.3. Use binary weighting and summation to plot the ADC’s output (using, for example, a nonlinear dependent source) or simply use the DAC’s from problem A4.1.

A4.3	Suppose two identical 3-bit ADCs with inputs tied together are clocked with a 100 MHz sampling frequency. Noting that the outputs of the ADC’s are identical show how two’s complement subtraction can be used to subtract the two outputs from each other with a constant result of VCM (= 0000 in two’s complement). Verify with LTspice.

A4.4	Re-derive Eqs. (4.7) – (4.13) for the accumulate-and-dump circuit seen in Fig. 4.12.

A4.5	Repeat Ex. 4.3 for a filter with a transfer function of

1-z-61-z-1

Also, plot the location of the filter’s poles and zeroes in the z-plane. Note that since the comb filter doesn’t use a number of delays that is a power of two (here we use K = 6) there will be a scale factor in the resulting filter’s transfer function. For this problem the word size, on the input of the integrator, is increased by log2(6) – 1 = 1.585 bits. We round this up to 2-bits, the same as a filter with K = 8. The result is that the magnitude response of the filter must be scaled by 6/8 or 0.75.

A4.6	Sketch the block diagram implementation and impulse response of the filter seen in Fig. 4.21.

A4.7	Repeat Ex. 4.7 if the transfer function of the filter is

1-z-41-z-2

Which equation in Ch. 4 describes the frequency response of this filter? Hint: it’s not Eq. (4.26)

A4.8	Repeat the example on page 140 with the values given using the topology seen in Fig. 4.30 with M = 2.

A4.9	Repeat the example on page 144 if K is increased to 16.