Code:

Machine Code:

20080001

20090001

01098020

01285022

210b0000

00000000

120a0003

014b5020

08100006

00000000

20080007

20090007

Assembly Code:

addi $t0, $0, 1		# t0 = 1

addi $t1, $0, 1		# t1 = 1

add $s0, $t0, $t1 	# s0 = t0 + t1 = 2

sub $t2, $t1, $t0 	# t2 = t1 - t0 = 0

addi $t3, $t0, 0	# t3 = t0 + (-1) = 1

nop

label:

beq $s0, $t2, end	# checks if SUM = 2

add $t2, $t2, $t3	# counter, where t2 + t3 = SUM + 1

j label				# if not equal, keep going through the counter

nop

end:

addi $t0, $0, 7		# t0 = 0 + 7 = 7

addi $t1, $0, 7		# t1 = 0 + 7 = 7

Verilog Code:

top.v

//-------------------------------------------------

// top.v

//-------------------------------------------------

module top_final(input         clk, reset,

//input  [31:0] readdata1, readdata2, //readdata is ReadDataM

output [31:0] writedata1, writedata2, dataadr1, dataadr2, //writedata is actually WriteDataM, and dataadr is the result of the ALU

output        MemWriteM1, MemWriteM2,

output [31:0] ResultW1, ResultW2); //This way the testbench tests for MemWrite at the memory stage

wire [31:0] pc;

wire [63:0] instr;

wire [31:0] readdata1, readdata2;

// instantiate processor and memories

mipssingle mipssingle(clk, reset, pc, instr, dataadr1, dataadr2,

writedata1, writedata2, readdata1, readdata2,

MemWriteM1, MemWriteM2, ResultW1, ResultW2);

//Instruction Memory

imem imem(pc[7:2], instr);

//Data Memory

dmem dmem(clk, MemWriteM1, MemWriteM2, dataadr1, dataadr2, writedata1, writedata2, //Changed memwrite to MemWriteM

readdata1, readdata2, reset);

endmodule

mipssingle.v

module mipssingle(input         clk, reset,

output [31:0] pcF,

input  [63:0] instrF,

output [31:0] ALUOutE1, ALUOutE2, WriteDataM1, WriteDataM2,

input  [31:0] readdataM1, readdataM2,

output		MemWriteM1, MemWriteM2,

output [31:0] ResultW1, ResultW2);

//Added MemWriteM1/MemWriteM2 for testbench checking

//Wires for 1st Path

wire        memtoregD1, memwriteD1;

wire        alusrcD1;

wire        regdstD1;

wire        regwriteD1, jump, pcsrcD1, zero;

wire [3:0]  alucontrolD1;

wire 		  EqualD1; // Output for the 'equals' module when solving for 'PCSrcD'

wire 		  branchD1;

wire [31:0] instrD1;

//Wires for 2nd Path

wire        memtoregD2, memwriteD2;

wire        alusrcD2;

wire        regdstD2;

wire        regwriteD2, pcsrcD2;

wire [3:0]  alucontrolD2;

wire 		  EqualD2; // Output for the 'equals' module when solving for 'PCSrcD'

wire 		  branchD2;

wire [31:0] instrD2;

wire temp;

assign temp = 1'b0;

//CONTROLLER UNIT 1 INSTANTIATION

controller1 c1(instrD1[31:26], instrD1[5:0], zero, //changed instr to instrd1

memtoregD1, memwriteD1, pcsrcD1,

alusrcD1, regdstD1, regwriteD1, jump,

alucontrolD1, EqualD1, branchD1); //EqualD1 and branch need to go in here now

//CONTROLLER UNIT 2 INSTANTIATION

controller2 c2(instrD2[31:26], instrD2[5:0], zero, //changed instr to instrd1

memtoregD2, memwriteD2, pcsrcD2,

alusrcD2, regdstD2, regwriteD2, temp,

alucontrolD2, EqualD2, branchD2); //EqualD1 and branch need to go in here now

datapath dp(clk, reset, memtoregD1, memtoregD2, pcsrcD1, pcsrcD2,

alusrcD1, alusrcD2, regdstD1, regdstD2, regwriteD1, regwriteD2,

jump, alucontrolD1, alucontrolD2,

zero, pcF, instrF[63:32], instrF[31:0],

ALUOutE1, ALUOutE2, WriteDataM1, WriteDataM2, readdataM1, readdataM2,

EqualD1, EqualD2, memwriteD1, memwriteD2, branchD1, branchD2,

MemWriteM1, MemWriteM2, instrD1, instrD2, ResultW1, ResultW2); //added EqualD1, memwrite, branch, MemWriteM1, and instrd1

endmodule

//CONTROLLER UNIT 1

module controller1(input  [5:0] op, funct,

input        zero,

output       memtoreg, memwrite,

output       pcsrc,

output       alusrc,

output       regdst,

output       regwrite,

output       jump,

output [3:0] alucontrol,  // 4 bits for SLL

input		   EqualD1,

output	   branch);

wire [1:0] aluop;

maindec md(op, memtoreg, memwrite, branch,

alusrc, regdst, regwrite, jump,

aluop);

aludec  ad(funct, aluop, alucontrol);

assign pcsrc = (branch & EqualD1); //For branching

endmodule

//CONTROLLER UNIT 2

module controller2(input  [5:0] op, funct,

input        zero,

output       memtoreg, memwrite,

output       pcsrc,

output       alusrc,

output       regdst,

output       regwrite,

output       jump,

output [3:0] alucontrol,  // 4 bits for SLL

input		   EqualD2,

output	   branch);

wire [1:0] aluop;

maindec md(op, memtoreg, memwrite, branch,

alusrc, regdst, regwrite, jump,

aluop);

aludec  ad(funct, aluop, alucontrol);

assign pcsrc = (branch & EqualD2); //For branching

endmodule

module maindec(input  [5:0] op,

output       memtoreg, memwrite,

output       branch,

output       alusrc,

output       regdst,

output       regwrite,

output       jump,

output [1:0] aluop);

reg [8:0] controls;

assign {regwrite, regdst, alusrc,

branch, memwrite,

memtoreg, jump, aluop}

= controls;

always @(*)

case(op)

6'b000000: controls <= 9'b110000010; //Rtype

6'b100011: controls <= 9'b101001000; //LW

6'b101011: controls <= 9'b001010000; //SW

6'b000100: controls <= 9'b000100001; //BEQ

6'b001000: controls <= 9'b101000000; //ADDI

6'b000010: controls <= 9'b000000100; //J

default:   controls <= 9'bxxxxxxxxx; //???

endcase

endmodule

module aludec(input      [5:0] funct,

input      [1:0] aluop,

output reg [3:0] alucontrol); // 4-bits for SLL

always @(*)

case(aluop)

2'b00: alucontrol <= 4'b0010;  // add

2'b01: alucontrol <= 4'b1010;  // sub

2'b11: alucontrol <= 4'b1011;  // slt

default: case(funct)           // RTYPE

6'b100000: alucontrol <= 4'b0010; // ADD

6'b100010: alucontrol <= 4'b1010; // SUB

6'b100100: alucontrol <= 4'b0000; // AND

6'b100101: alucontrol <= 4'b0001; // OR

6'b101010: alucontrol <= 4'b1011; // SLT

//6'b000010: alucontrol <= 4'b0101; // MUL

//6'b011010: alucontrol <= 4'b0110; // DIV

default:   alucontrol <= 4'bxxxx; // ???

endcase

endcase

endmodule

module datapath(input         clk, reset,

input         memtoregD1, memtoregD2, pcsrcD1, pcsrcD2,

input         alusrcD1, alusrcD2, //AluSrcD

input         regdstD1, regdstD2, //RegDstD

input         regwriteD1, regwriteD2, jump, //RegWriteD

input  [3:0]  alucontrolD1, alucontrolD2, //ALUControlD

output        zero, //trash wire (not used)

output [31:0] pcF,

input  [31:0] instrF1, //1st Instruction

input  [31:0] instrF2, //2nd Instruction

output [31:0] ALUOutM1, ALUOutM2, WriteDataM1, WriteDataM2,

input  [31:0] ReadDataM1, ReadDataM2,

output		  EqualD1, EqualD2,

input		  memwriteD1, memwriteD2,

input 		  BranchD1, BranchD2,

output		  MemWriteM1, MemWriteM2,

output [31:0] instrD1,

output [31:0] instrD2,

output [31:0] ResultW1, ResultW2);

wire [31:0] pcnext, pcnextbr, pcplus8F, pcbranch, pcplus4F, pcplus4D;

wire [31:0] signimmD1, signimmD2, signimmshD1, signimmshD2;

wire [31:0] srcaD1, srcaD2, srcbD1, srcbD2;

//wire [31:0] ResultW1, ResultW2;

/*************************************************************/

// Extra wires created for Pipeline/Hazard-Unit

/*************************************************************/

wire MemtoRegE1, MemWriteE1, RegWriteE1, MemtoRegM1, RegWriteM1, RegWriteW1, StallF, StallD1, StallD2, FlushE1, FlushE2, ForwardAD1, ForwardBD1, ALUSrcE1, RegDstE1, MemtoRegW1;

wire MemtoRegE2, MemWriteE2, RegWriteE2, MemtoRegM2, RegWriteM2, RegWriteW2, ForwardAD2, ForwardBD2, ALUSrcE2, RegDstE2, MemtoRegW2;

wire [4:0] RsD1, RtD1, WriteRegE1, WriteRegM1, WriteRegW1, RsE1, RtE1, RdE1, shamt1;

wire [4:0] RsD2, RtD2, WriteRegE2, WriteRegM2, WriteRegW2, RsE2, RtE2, RdE2, shamt2;

wire [2:0] ForwardAE1, ForwardBE1;

wire [2:0] ForwardAE2, ForwardBE2;

wire [31:0] srcaeqD1, srcbeqD1, SrcAE1, SrcAEM1, SrcBE1, SrcBEMM1, SignImmE1, WriteDataE1, ALUOutE1, ReadDataW1, ALUOutW1, pcplus8D;

wire [31:0] srcaeqD2, srcbeqD2, SrcAE2, SrcAEM2, SrcBE2, SrcBEMM2, SignImmE2, WriteDataE2, ALUOutE2, ReadDataW2, ALUOutW2;

wire [3:0]  ALUControlE1;

wire [3:0]  ALUControlE2;

/*************************************************************/

/*************************************************************/

// Hazard Unit

/*************************************************************/

hazunit hu(BranchD1, BranchD2, MemtoRegE1, MemtoRegE2, RegWriteE1, RegWriteE2, MemtoRegM1, MemtoRegM2,

RegWriteM1, RegWriteM2, RegWriteW1, RegWriteW2,

RsD1, RsD2, RtD1, RtD2,

RsE1, RsE2, RtE1, RtE2,

WriteRegE1, WriteRegE2, WriteRegM1, WriteRegM2, WriteRegW1, WriteRegW2,

StallF, StallD1, StallD2, FlushE1, FlushE2,

ForwardAD1, ForwardAD2, ForwardBD1, ForwardBD2,

ForwardAE1, ForwardBE1, ForwardAE2, ForwardBE2); //Hazard Unit instantiation

/*************************************************************/

/*************************************************************/

// Pipeline Registers & MUX's & Equalizers added

/*************************************************************/

//Pipeline register in between Fetch and Decode

mux2  #(32) eqda1(srcaD1, ALUOutM1, //ForwardAD1 MUX in Decode stage

ForwardAD1, srcaeqD1);

mux2  #(32) eqdb1(srcbD1, ALUOutM1, //ForwardBD1 MUX in Decode stage

ForwardBD1, srcbeqD1);

equals      eq1(srcaeqD1, srcbeqD1, //Checks to see if srcad1 and srcb are equal to determine branching

EqualD1);

mux2  #(32) eqda2(srcaD2, ALUOutM2, //ForwardAD1 MUX in Decode stage

ForwardAD2, srcaeqD2);

mux2  #(32) eqdb2(srcbD2, ALUOutM2, //ForwardBD1 MUX in Decode stage

ForwardBD2, srcbeqD2);

equals      eq2(srcaeqD2, srcbeqD2, //Checks to see if srcad1 and srcb are equal to determine branching

EqualD2);

//FIRST LANE

regf	/*#(64)*/ rfe(clk, (pcsrcD1 | jump), /*(pcsrcD2 | jump),*/ reset, ~StallD1/*enable*/, instrF1, pcplus8F, //inputs into register f

instrD1, pcplus8D); //outputs out of register f

//Pipeline register in between Decode and Execute

regd	/*#(120)*/rd(clk, FlushE1, reset, regwriteD1, memtoregD1, memwriteD1, alucontrolD1, alusrcD1, regdstD1, srcaD1, srcbD1,

instrD1[25:21], instrD1[20:16], instrD1[15:11], signimmD1, instrD1[10:6], //inputs into register d

RegWriteE1, MemtoRegE1, MemWriteE1, ALUControlE1, ALUSrcE1, RegDstE1, SrcAE1, SrcBE1,

RsE1, RtE1, RdE1, SignImmE1, shamt1); //outputs out of register d

//Pipeline register in between Execute and Memory

rege	/*#(71)*/re(clk, reset, RegWriteE1, MemtoRegE1, MemWriteE1, ALUOutE1, WriteDataE1, WriteRegE1, //inputs into register e

RegWriteM1, MemtoRegM1, MemWriteM1, ALUOutM1, WriteDataM1, WriteRegM1); //output out of register e

//Pipeline register in between Memory and Write Back

regm	/*#(71)*/rm(clk, reset, RegWriteM1, MemtoRegM1, ReadDataM1, ALUOutM1, WriteRegM1, //inputs into register m

RegWriteW1, MemtoRegW1, ReadDataW1, ALUOutW1, WriteRegW1); //outputs out of register m

//SECOND LANE

//Pipeline register in between Fetch and Decode

regf	/*#(64)*/ rfe2(clk, (pcsrcD1 | jump), /*(pcsrcD2 | jump),*/ reset, ~StallD2/*enable*/, instrF2, pcplus4F, //inputs into register f

instrD2, pcplus4D); //outputs out of register f

//Pipeline register in between Decode and Execute

regd	/*#(120)*/rd2(clk, FlushE2, reset, regwriteD2, memtoregD2, memwriteD2, alucontrolD2, alusrcD2, regdstD2, srcaD2, srcbD2,

instrD2[25:21], instrD2[20:16], instrD2[15:11], signimmD2, instrD2[10:6], //inputs into register d

RegWriteE2, MemtoRegE2, MemWriteE2, ALUControlE2, ALUSrcE2, RegDstE2, SrcAE2, SrcBE2,

RsE2, RtE2, RdE2, SignImmE2, shamt2); //outputs out of register d

//Pipeline register in between Execute and Memory

rege	/*#(71)*/re2(clk, reset, RegWriteE2, MemtoRegE2, MemWriteE2, ALUOutE2, WriteDataE2, WriteRegE2, //inputs into register e

RegWriteM2, MemtoRegM2, MemWriteM2, ALUOutM2, WriteDataM2, WriteRegM2); //output out of register e

//Pipeline register in between Memory and Write Back

regm	/*#(71)*/rm2(clk, reset, RegWriteM2, MemtoRegM2, ReadDataM2, ALUOutM2, WriteRegM2, //inputs into register m

RegWriteW2, MemtoRegW2, ReadDataW2, ALUOutW2, WriteRegW2); //outputs out of register m

/*************************************************************/

//The below is orginal code

// next PC logic

flopenr #(32) pcreg(clk, reset, ~StallF/*enable*/,pcnext, pcF);

adder       pcadd1(pcF, 32'b1000, pcplus8F);

adder       pcadd4(pcF, 32'b100, pcplus4F);

sl2         immsh(signimmD1, signimmshD1);

adder       pcadd2(pcplus4D, signimmshD1, pcbranch);

mux2 #(32)  pcbrmux(pcplus8F, pcbranch, pcsrcD1,

pcnextbr);

mux2 #(32)  pcmux(pcnextbr, {pcplus8F[31:28],

instrD1[25:0], 2'b00},

jump, pcnext);

// register file logic

regfile     rf(clk, RegWriteW1, RegWriteW2, instrD1[25:21], instrD2[25:21],

instrD1[20:16], instrD2[20:16], WriteRegW1, WriteRegW2,

ResultW1, ResultW2,

srcaD1, srcaD2, srcbD1, srcbD2);

mux2 #(5)   wrmux1(RtE1, RdE1,

RegDstE1, WriteRegE1);

mux2 #(5)   wrmux2(RtE2, RdE2,

RegDstE2, WriteRegE2);

mux2 #(32)  resmux1(ALUOutW1, ReadDataW1,

MemtoRegW1, ResultW1);

mux2 #(32)  resmux2(ALUOutW2, ReadDataW2,

MemtoRegW2, ResultW2);

signext     se1(instrD1[15:0], signimmD1);

signext     se2(instrD2[15:0], signimmD2);

// ALU1 logic

mux5 #(32)  srcaemux1(SrcAE1, ResultW1, ALUOutM1, ResultW2, ALUOutM2,

ForwardAE1, SrcAEM1);

mux5 #(32)  srcbemux1(SrcBE1, ResultW1, ALUOutM1, ResultW2, ALUOutM2,

ForwardBE1, WriteDataE1);

mux2 #(32)  srcbemmux1(WriteDataE1, SignImmE1,

ALUSrcE1, SrcBEMM1);

alu         alu1(SrcAEM1, SrcBEMM1, ALUControlE1, shamt1,  // SLL

ALUOutE1, zero);

// ALU2 logic

mux5 #(32)  srcaemux2(SrcAE2, ResultW2, ALUOutM2, ResultW1, ALUOutM1,

ForwardAE2, SrcAEM2);

mux5 #(32)  srcbemux2(SrcBE2, ResultW2, ALUOutM2, ResultW1, ALUOutM1,

ForwardBE2, WriteDataE2);

mux2 #(32)  srcbemmux2(WriteDataE2, SignImmE2,

ALUSrcE2, SrcBEMM2);

alu         alu2(SrcAEM2, SrcBEMM2, ALUControlE2, shamt2,  // SLL

ALUOutE2, zero);

Endmodule

mipsmem.v

//------------------------------------------------

// mipsmem.v

//------------------------------------------------

module dmem(input         clk, we1, we2,

input  [31:0] a1, a2, wd1, wd2,

output [31:0] rd1, rd2,

input 		  reset);

reg  [31:0] RAM[63:0];

assign rd1 = RAM[a1[31:2]]; // word aligned for address 1

assign rd2 = RAM[a2[31:2]]; // word aligned for address 2

always @(posedge clk)

begin

if (we1)

RAM[a1[31:2]] <= wd1;

if (we2)

RAM[a2[31:2]] <= wd2;

end

endmodule

module imem(input  [5:0] a,

output [63:0] rd); //Expanded from 31:0 to 63:0

reg  [31:0] RAM[63:0];

initial

begin

$readmemh("memfile.dat",RAM);

end

assign rd = {RAM[a], RAM[a+1]}; // word aligned to take TWO instructions at once

endmodule

mipsparts.v

//------------------------------------------------

// mipsparts.v

//------------------------------------------------

module regfile(input         clk,

input         we3,

input		 we4,

input  [4:0]  ra1, ra3, ra2, ra4, wa3, wa4,

input  [31:0] wd3, wd4,

output [31:0] rd1, rd3, rd2, rd4);

reg [31:0] rf[31:0];

// three ported register file

// read two ports combinationally

// write third port on rising edge of clock

// register 0 hardwired to 0

always @(negedge clk)

begin

if (we3) rf[wa3] <= wd3;

if (we4) rf[wa4] <= wd4;

end

assign rd1 = (ra1 != 0) ? rf[ra1] : 0;

assign rd2 = (ra2 != 0) ? rf[ra2] : 0;

assign rd3 = (ra3 != 0) ? rf[ra3] : 0;

assign rd4 = (ra4 != 0) ? rf[ra4] : 0;

endmodule

module adder(input [31:0] a, b,

output [31:0] y);

assign y = a + b;

endmodule

module sl2(input  [31:0] a,

output [31:0] y);

// shift left by 2

assign y = {a[29:0], 2'b00};

endmodule

module signext(input  [15:0] a,

output [31:0] y);

assign y = {{16{a[15]}}, a};

endmodule

module flopr #(parameter WIDTH = 8)

(input                  clk, reset,

input      [WIDTH-1:0] d,

output reg [WIDTH-1:0] q);

always @(posedge clk)

if (reset) q <= 0;

else       q <= d;

endmodule

module flopenr #(parameter WIDTH = 8)

(input                  clk, reset,

input                  en,

input      [WIDTH-1:0] d,

output reg [WIDTH-1:0] q);

always @(posedge clk, posedge reset)

if      (reset) q <= 0;

else if (en)    q <= d;

endmodule

module mux2 #(parameter WIDTH = 8)

(input  [WIDTH-1:0] d0, d1,

input              s,

output [WIDTH-1:0] y);

assign y = s ? d1 : d0;

endmodule

// mux3 needed for LUI

module mux3 #(parameter WIDTH = 8)

(input  [WIDTH-1:0] d0, d1, d2,

input  [1:0]       s,

output [WIDTH-1:0] y);

assign #1 y = s[1] ? d2 : (s[0] ? d1 : d0);

endmodule

// mux5 needed for Superscalar forwarding

module mux5 #(parameter WIDTH = 8)

(input  [WIDTH-1:0] d0, d1, d2, d3, d4,

input  [2:0]       s,

output [WIDTH-1:0] y);

assign #1 y = s[2] ? d4 : (s[1] & s[0]) ? d3 : s[1] ? d2 : s[0] ? d1 : d0;

//if select = 100, then out d4; else if select = 011, then out d3; else if select = 10, then out d2; else if select = 1, then out d1; else if select = 0, then out d0

endmodule

/*************************************************************/

// Pipeline Registers

/*************************************************************/

//Pipeline register in between Fetch and Decode

module regf (input 				clk, clr, reset, en,

input 		[31:0]	instrF1, pcplus4f,

output reg [31:0]	instrD1, pcplus4d);

always @(posedge clk) //Everything stays synchronous to clock

if (en) //So long as there's an enable (no StallF), continue through register operations

if (clr || reset) begin   //If clear or reset, set all outputs as 0

instrD1 <= 0;

pcplus4d <= 0;

end

else begin  //If no reset, let values go through as clock is high

instrD1 <= instrF1;

pcplus4d <= pcplus4f;

end

endmodule

//Pipeline register in between Decode and Execute

module regd (input 	 		   clk, clr, reset, regwriteD1,

memtoregD1, memwriteD1,

input  	[3:0]  alucontrolD1,

input 	  		   alusrcD1, regdstD1,

input 		[31:0] srcaD1, srcbD1,

input 		[4:0]  rsdD1, rtdD1, rddD1,

input 		[31:0] signimmD1,

input		[4:0]  aluinstrD1,

output reg	 	   RegWriteE1, MemtoRegE1,

MemWriteE1,

output reg [3:0]  ALUControlE1,

output	reg	   	   ALUSrcE1, RegDstE1,

output reg [31:0] SrcAE1, SrcBE1,

output reg [4:0]  RsE1, RtE1, RdE1,

output reg [31:0] SignImmE1,

output reg [4:0]  shamt1);

always @(posedge clk) //Everything stays synchronous to clock

if (clr || reset) begin   //If clear or reset, set all outputs as 0

RegWriteE1 <= 0;

MemtoRegE1 <= 0;

MemWriteE1 <= 0;

ALUControlE1 <= 0;

ALUSrcE1 <= 0;

RegDstE1 <= 0;

SrcAE1 <= 0;

SrcBE1 <= 0;

RsE1 <= 0;

RtE1 <= 0;

RdE1 <= 0;

SignImmE1 <= 0;

shamt1 <= 0;

end

else begin   //If no reset, let values go through as clock is high

RegWriteE1 <= regwriteD1;

MemtoRegE1 <= memtoregD1;

MemWriteE1 <= memwriteD1;

ALUControlE1 <= alucontrolD1;

ALUSrcE1 <= alusrcD1;

RegDstE1 <= regdstD1;

SrcAE1 <= srcaD1;

SrcBE1 <= srcbD1;

RsE1 <= rsdD1;

RtE1 <= rtdD1;

RdE1 <= rddD1;

SignImmE1 <= signimmD1;

shamt1 <= aluinstrD1;

end

endmodule

//Pipeline register in between Execute and Memory

module rege (input 			   clk, reset, RegWriteE1, MemtoRegE1, MemWriteE1,

input 		[31:0] ALUOutE1, WriteDataE1,

input 		[4:0]  WriteRegE1,

output	reg		   RegWriteM1, MemtoRegM1, MemWriteM1,

output reg [31:0] ALUOutM1, WriteDataM1,

output reg [4:0]  WriteRegM1);

always @(posedge clk) //Everything stays synchronous to clock

if (reset)

begin   //If reset is high, set all outputs 0

RegWriteM1 <= 0;

MemtoRegM1 <= 0;

MemWriteM1 <= 0;

ALUOutM1 <= 0;

WriteDataM1 <= 0;

WriteRegM1 <= 0;

end

else begin //If no reset, let values go through as clock is high

RegWriteM1 <= RegWriteE1;

MemtoRegM1 <= MemtoRegE1;

MemWriteM1 <= MemWriteE1;

ALUOutM1 <= ALUOutE1;

WriteDataM1 <= WriteDataE1;

WriteRegM1 <= WriteRegE1;

end

endmodule

//Pipeline register in between Memory and Write Back

module regm (input 			   clk, reset, RegWriteM1,

MemtoRegM1,

input 		[31:0] ReadDataM1, ALUOutM1,

input 		[4:0]  WriteRegM1,

output	reg		   RegWriteW1, MemtoRegW1,

output reg [31:0] ReadDataW1, ALUOutW1,

output reg [4:0]  WriteRegW1);

always @(posedge clk) //Everything stays synchronous to clock

if (reset) //When reset is high, give all outputs 0

begin

RegWriteW1 <= 0;

MemtoRegW1 <= 0;

ReadDataW1 <= 0;

ALUOutW1 <= 0;

WriteRegW1 <= 0;

end

else //If no reset, let values go through as clock is high

begin

RegWriteW1 <= RegWriteM1;

MemtoRegW1 <= MemtoRegM1;

ReadDataW1 <= ReadDataM1;

ALUOutW1 <= ALUOutM1;

WriteRegW1 <= WriteRegM1;

end

endmodule

/*************************************************************/

// Equalizer

/*************************************************************/

//This is the arithmetic that will return whether or not srca and srcb are equal. Then follow up on the branch instruction

module equals (input [31:0] A, B,

output Y);

assign Y = ( A === B ) ? 1 : 0; //If srca is equal to srcb, then output 1 bit 1

endmodule

mipshaz.v

module hazunit (input 		 BranchD1, BranchD2, MemtoRegE1, MemtoRegE2, RegWriteE1, RegWriteE2, MemtoRegM1, MemtoRegM2,

input        RegWriteM1, RegWriteM2, RegWriteW1, RegWriteW2,

input  [4:0] RsD1, RsD2, RtD1, RtD2,

input  [4:0] RsE1, RsE2, RtE1, RtE2,

input  [4:0] WriteRegE1, WriteRegE2, WriteRegM1, WriteRegM2, WriteRegW1, WriteRegW2,

output 		 StallF, StallD1, StallD2, FlushE1, FlushE2,

output		 ForwardAD1, ForwardAD2, ForwardBD1, ForwardBD2,

output [2:0] ForwardAE1, ForwardBE1,

output [2:0] ForwardAE2, ForwardBE2);

wire branchstall1, lwstall1; //Wires used for the stalls and flush

wire branchstall2, lwstall2;

//Forwarding Logic

assign ForwardAD1 = (RsD1 != 0) & (RsD1 == WriteRegM1) & RegWriteM1; //Logic for ForwardAD to push forward values from memory to decode or just pass through

assign ForwardBD1 = (RtD1 != 0) & (RtD1 == WriteRegM1) & RegWriteM1; //Logic for ForwardBD to push forward values from memory to decode or just pass through

assign ForwardAD2 = (RsD2 != 0) & (RsD2 == WriteRegM2) & RegWriteM2; //Logic for ForwardAD to push forward values from memory to decode or just pass through

assign ForwardBD2 = (RtD2 != 0) & (RtD2 == WriteRegM2) & RegWriteM2; //Logic for ForwardBD to push forward values from memory to decode or just pass through

//As long as it's an I-Type (because branching has to use two registers RS and RT) AND the same register is about to be reused before placing the value in RegFile AND we still need to write the previous value of an older instruction into RegFile

//lane 1

assign ForwardAE1 = ((RsE1 != 0) & (RsE1 == WriteRegM1) & RegWriteM1) ? 3'b010 : ((RsE1 != 0) & (RsE1 == WriteRegW1) & RegWriteW1) ? 3'b001 : //2'b00 ); //Logic for ForwardAE to push forward values from memory to execute or WB to execute

((RsE1 != 0) & (RsE1 == WriteRegM2) & RegWriteM2) ? 3'b100 : (((RsE1 != 0) & (RsE1 == WriteRegW2) & RegWriteW2) ? 3'b011 : 3'b000); //forwarding from lane 1 to lane 2

assign ForwardBE1 = ((RtE1 != 0) & (RtE1 == WriteRegM1) & RegWriteM1) ? 3'b010 : ((RtE1 != 0) & (RtE1 == WriteRegW1) & RegWriteW1) ? 3'b001 : //2'b00 ); //Logic for ForwardBE to push forward values from memory to execute or WB to execute

((RtE1 != 0) & (RtE1 == WriteRegM2) & RegWriteM2) ? 3'b100 : (((RtE1 != 0) & (RtE1 == WriteRegW2) & RegWriteW2) ? 3'b011 : 3'b000); //forwarding from lane 1 to lane 2

//lane 2

assign ForwardAE2 = ((RsE2 != 0) & (RsE2 == WriteRegM2) & RegWriteM2) ? 3'b010 : ((RsE2 != 0) & (RsE2 == WriteRegW2) & RegWriteW2) ? 3'b001 : //2'b00 ); //Logic for ForwardAE to push forward values from memory to execute or WB to execute

((RsE2 != 0) & (RsE2 == WriteRegM1) & RegWriteM1) ? 3'b100 : (((RsE2 != 0) & (RsE2 == WriteRegW1) & RegWriteW1) ? 3'b011 : 3'b000); //forwarding from lane 1 to lane 2

assign ForwardBE2 = ((RtE2 != 0) & (RtE2 == WriteRegM2) & RegWriteM2) ? 3'b010 : ((RtE2 != 0) & (RtE2 == WriteRegW2) & RegWriteW2) ? 3'b001 : //2'b00 ); //Logic for ForwardBE to push forward values from memory to execute or WB to execute

((RtE2 != 0) & (RtE2 == WriteRegM1) & RegWriteM1) ? 3'b100 : (((RtE2 != 0) & (RtE2 == WriteRegW1) & RegWriteW1) ? 3'b011 : 3'b000); //forwarding from lane 1 to lane 2

//default = 000, Grab from WB2 = 001, Grab from Mem2 = 010, Grab from WB1 = 011, Grab from Mem1 = 100

//assign ForwardBE1 = ((RtE1 != 0) & (RtE1 == WriteRegM1) & RegWriteM1) ? 2'b10 : ( ((RtE1 != 0) & (RtE1 == WriteRegW1) & RegWriteW1) ? 2'b01 : 2'b00 ); //Logic for ForwardBE to push forward values from memory to execute or WB to execute

//assign ForwardAE2 = ((RsE2 != 0) & (RsE2 == WriteRegM2) & RegWriteM2) ? 2'b10 : ( ((RsE2 != 0) & (RsE2 == WriteRegW2) & RegWriteW2) ? 2'b01 : 2'b00 ); //Logic for ForwardAE to push forward values from memory to execute or WB to execute

//assign ForwardBE2 = ((RtE2 != 0) & (RtE2 == WriteRegM2) & RegWriteM2) ? 2'b10 : ( ((RtE2 != 0) & (RtE2 == WriteRegW2) & RegWriteW2) ? 2'b01 : 2'b00 ); //Logic for ForwardBE to push forward values from memory to execute or WB to execute

//IF rs and/or rt holds a value AND the older rs and rt was used in a previous instruction (memory stage) AND the previous instruction is going to write a value to the register file

//THEN grabs the 'previous' instructions value

//ELSE IF rs and/or rt holds a value AND the older rs and rt was used in 2 previous instructions ago (write back stage) AND that previous instruction is going to write a value to the register file

//THEN grabs the value of the '2nd previous' instruction value

//ELSE 'no forwarding'

//Stalling Logic

assign branchstall1 = (BranchD1 & (RegWriteE1 & ((WriteRegE1 == RsD1) | (WriteRegE1 == RtD1)) | MemtoRegM1 & ((WriteRegM1 == RsD1) | (WriteRegM1 == RtD1))));

assign branchstall2 = (BranchD2 & (RegWriteE2 & ((WriteRegE2 == RsD2) | (WriteRegE2 == RtD2)) | MemtoRegM2 & ((WriteRegM2 == RsD2) | (WriteRegM2 == RtD2)))); //Logic to check if branching

assign lwstall1 = (((RsD1 == RtE1) | (RtD1 == RtE1)) & MemtoRegE1);

assign lwstall2 = (((RsD2 == RtE2) | (RtD2 == RtE2)) & MemtoRegE2); //Logic to check if load word

//assign parallelstall = (RsE2 == WriteRegM1) | (RsE1 == WriteRegM2);

assign StallF = lwstall1 | branchstall1 | lwstall2 | branchstall2; //Logic for StallF1

assign StallD1 = lwstall1 | branchstall1; //Logic for StallD

assign StallD2 = lwstall2 | branchstall2 | branchstall1; //Logic for StallD

assign FlushE1 = lwstall1 | branchstall1; //Logic for FlushE

assign FlushE2 = lwstall2 | branchstall2 | branchstall1; //Logic for FlushE

endmodule

alu32.v

//------------------------------------------------

// alu32.v

//------------------------------------------------

`timescale 1ns / 1ps

module alu(	input [31:0] A, B,

input [3:0] F, input [4:0] shamt, // SLL

output reg [31:0] Y, output Zero);

wire [31:0] S, Bout;

wire ltez;

assign Bout = F[3] ? ~B : B;

assign S = A + Bout + F[3];  // SLL

always @ ( * )

case (F[2:0])

3'b000: Y <= A & Bout;

3'b001: Y <= A | Bout;

3'b010: Y <= S;

3'b011: Y <= S[31];

3'b100: Y <= (Bout << shamt);  // SLL

3'b101: Y <= A * B; // MUL

3'b110: Y <= A / B; // DIV

endcase

assign Zero = (Y == 32'b0);

assign ltez = Zero | S[31];  // BLEZ

//	assign Overflow =  A[31]& Bout[31] & ~Y[31] |

//							~A[31] & ~Bout[31] & Y[31];

endmodule

mipstest.v

//------------------------------------------------

// mipstest.v

//------------------------------------------------

module testbench();

reg         clk;

reg         reset;

wire [31:0] dataadr1, dataadr2, writedata1, writedata2, readdata1, readdata2, ResultW1, ResultW2;

wire memwrite1, memwrite2;

// instantiate device to be tested

top_final dut(clk, reset, writedata1, writedata2,

dataadr1, dataadr2, memwritem1, memwritem2, ResultW1, ResultW2);

// initialize test

initial

begin

reset <= 1; # 22; reset <= 0;

end

// generate clock to sequence tests

always

begin

clk <= 1; # 5; clk <= 0; # 5;

end

endmodule